Part 1

- Gravity is a force that acts at a distance. Name two other forces that act at a distance.

- Name three contact forces.

- A boy has a mass of 40 kg.  Calculate the boy’s weight. Take g = 10 N/kg.

- What is a scalar quantity?

- Explain how a car can be moving at a constant speed but have changing velocity.

- State whether the following quantities are scalars or vectors:

acceleration	mass	   momentum	  time

- Name a piece of scientific equipment that you would use to find the weight of a block in a science laboratory.

- The object below has two forces acting on it, shown by the arrows.

Draw an arrow to show the resultant force on the object

- Calculate the resultant force acting on the objects below:

Part 2 (physics only)

- State the equation used to find the moment of a force.

- A 30 cm long spanner is used to undo a nut. A force of 20 N is applied to the end of the spanner. Calculate the moment of force applied to the spanner.

- Two children sit on a see-saw on opposite sides of the pivot. One child has a weight of 340 N and sits 1.2 m from the pivot. If the other child has a weight of 420 N how far does this child need to sit from the pivot for the see-saw to be balanced?

- A crowbar is used to lift up a floor board.  The crowbar has a length of 40 cm from the pivot to the end of the crowbar, and the distance from the bend to the lifting point is 12 cm. If the force applied to the end of the crowbar is 300 N,  work out the size of the force applied to the floor board.

- A box with a weight of 400 N is raised using a lever 2 m long.  The lever rotates around a pivot 50 cm from the lifting end of the lever.  Work out the force applied to the end of the lever.

- The 30 tooth large cog is made to rotate in a clockwise direction.  In which direction will the smaller cog rotate?

- In the gear system shown above, the 30 tooth cog rotates once every 5 seconds.  The smaller cog has 20 teeth.  Calculate how long it will take to the smaller cog to complete one revolution.

- Look at the following gear system. In which direction will the yellow cog rotate if the red cog is made to rotate anti-clockwise?